Finite State Machine

Q1.	The state table of a finite-state machine (FSM) with one input w and 2 outputs z1 and z2 is given below:

| Present State | Next State |  | Output |  |
| --- | --- | --- | --- | --- |
|  | w=0 | w=1 | z1 | zo |
| S0 | S2 | S1 | 0 | 0 |
| S1 | S2 | S1 | 0 | 1 |
| S2 | S2 | S3 | 1 | 0 |
| S3 | S1 | S3 | 0 | 0 |

- Explain weather the given FSM is a Moore-type or Mealy-type state machine?

- The given FSM is to be implemented as a synchronous sequential circuit with T flip-flops using the state assignments: S0=00, S1=01, S2=10, S3=11. Derive the equations for the inputs to the T flip-flops, and the equations for the outputs z1 and zo.

Q2. The state diagram for a finite state machine (FSM) with one input w and two outputs z2 and z1 is given below:

- Does the above state diagram use a Moore or Mealy-type model to represent the FSM? Explain your answer.

- What is the minimum number of state variables required to represent the sates.

- Using the state-assignment: A=00, B=01, C=11, and D=10, develop the next state and output equation for implementing the FSM.

Q3. In the circuit below:

- Is the design a Moore or Mealy Model?

- Derive the state table assuming the states are: S0=00, S1=01, S2=10, S3=11.

- Draw the state diagram showing the state, input, and output.

Q4.		Consider the following circuit which implements a finite-state machine (FSM):

- Derive the state assigned table for the FSM.

- Complete the following timing diagram for the circuit by assuming Q1=Q0=0 at the beginning.

- Derive the state assigned table if JK flip-flops (instead of D flip-flops) are to be used to implement the FSM.

Q5. Given the following logic circuit, derive its state table and state diagram. If the following sequence 1010110101 is applied to the x input of the circuit with initial state 01, determine the resulting output sequence on z output.

Q6.  Given is the following timing diagram, clock signal, and input waveforms:

- Derive the state-assigned table.

- Draw the state diagram.

- Derive the circuit that implement the FSM.